OPC-UA TAG REGISTER &

SCADA INTEGRATION ARCHITECTURE

Lube Oil Blending Plant — Document No: SCA-001 Rev 2.1

# 1. System Architecture Overview

The SCADA/IoT architecture follows a four-layer model: Field Layer (PLCs, sensors, actuators), Communication Layer (OPC-UA over TCP/IP, Ethernet), SCADA/HMI Layer (Ignition SCADA v8.1), and Cloud/Analytics Layer (Azure IoT Hub + Power BI). All inter-layer communication uses OPC-UA (IEC 62541) to ensure vendor-neutral, secure data exchange.

| Layer | Components & Details |
| --- | --- |
| Field Layer | 3× Siemens S7-1500 PLCs (PLC-01, PLC-02, PLC-03), 1× Safety PLC (Siemens F-CPU). I/O: 248 analog inputs, 312 digital I/O. HART-enabled sensors throughout. |
| Communication | Industrial Ethernet (PROFINET). OPC-UA Server: Siemens S7 OPC-UA integrated server on each PLC. Baud: 100 Mbps. Polling interval: 1 second for process variables, 100 ms for safety. |
| SCADA / HMI | Ignition SCADA v8.1 by Inductive Automation. OPC-UA Client connects to all 3 PLCs. Tag historian: Ignition built-in + InfluxDB mirror. HMI screens: 14 (overview, per-vessel, tank farm, utilities, alarms). |
| Edge / Gateway | Dell Edge Gateway 3002 running Ignition Edge. Local buffering: 72 hours. MQTT Sparkplug-B to Azure IoT Hub. TLS 1.3 encryption. |
| Cloud / Analytics | Azure IoT Hub → Azure Stream Analytics → Azure Data Lake. Power BI dashboards for OEE, batch yield, alarm analytics. Retention: 7 years. |
| Historian | Canary Labs Axiom Historian. 1-second resolution. Connects to SAP via REST API for batch event correlation. |

# 2. OPC-UA Address Space Model

The OPC-UA address space follows the ISA-95 hierarchy: Plant → Area → Unit → Module → Tag. All nodes are typed using the OPC-UA Information Model and comply with OPC UA for Process Control (OPC 10000-9).

Server Endpoint: opc.tcp://10.1.10.50:4840 (PLC-01), 10.1.10.51:4840 (PLC-02), 10.1.10.52:4840 (PLC-03)

Security Policy: Basic256Sha256 / SignAndEncrypt — mandatory for all clients

Authentication: X.509 certificate-based. No anonymous access permitted.

Namespace: urn:lobp:plant:scada:v2

Node naming convention: ns=2;s=LOBP.<Area>.<UnitID>.<TagFunction>.<Suffix>  Example: ns=2;s=LOBP.BlendingHall.V101.Temperature.PV

## 2.1  OPC-UA Tag Register — Process Variables

| Tag ID | OPC-UA Node Path | Unit | Type | Range | Accuracy | PLC / I/O Channel | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V101-TI-01 | V101/Temperature/Inlet | °C | Analog | 30.0–85.0 | ±0.5°C | PLC-01 / Slot 3, Ch 1 | Active |
| V101-LI-01 | V101/Level/PV | % fill | Analog | 0–100 | ±0.2% | PLC-01 / Slot 3, Ch 2 | Active |
| V101-FI-01 | V101/FlowRate/In | L/min | Analog | 0–1,200 | ±0.5% | PLC-01 / Slot 4, Ch 1 | Active |
| V101-FT-01 | V101/FlowTotaliser/In | L | Analog | 0–9,999,999 | — | PLC-01 / Slot 4, Ch 2 | Active |
| V101-AG-01 | V101/Agitator/Speed | RPM | Analog | 0–120 | ±1 RPM | PLC-01 / Slot 5, Ch 1 | Active |
| V101-AG-01-RUN | V101/Agitator/RunStatus | Bool | Digital | 0/1 | — | PLC-01 / Slot 5, Ch 2 | Active |
| P201-DI-01 | P201/DifferentialPressure | bar | Analog | 0–10 | ±0.01 bar | PLC-02 / Slot 2, Ch 1 | Active |
| P201-ST-01 | P201/PumpStatus/Run | Bool | Digital | 0/1 | — | PLC-02 / Slot 2, Ch 2 | Active |
| T08-LI-01 | T08/Level/PV | % fill | Analog | 0–100 | ±0.2% | PLC-03 / Slot 1, Ch 1 | Active |
| T08-TI-01 | T08/Temperature/PV | °C | Analog | 0–80 | ±0.5°C | PLC-03 / Slot 1, Ch 2 | Active |
| V102-PI-01 | V102/Pressure/Vessel | barg | Analog | 0–2.0 | ±0.01 bar | PLC-02 / Slot 3, Ch 1 | Active |
| PLANT-AL-01 | Plant/Alarm/FireGas | Bool | Digital | 0/1 | — | Safety PLC | Active |

# 3. Alarm & Event Configuration

Alarm philosophy follows ISA-18.2. Priority levels are: Priority 1 (Safety/Emergency — immediate action), Priority 2 (Critical Process — action within 5 min), Priority 3 (Warning — action within 30 min), Priority 4 (Advisory — log only).

| Alarm Tag | Description | Setpoint | Priority | Action Required |
| --- | --- | --- | --- | --- |
| V101-LI-01-HH | Blender V101 Level High-High | 95% fill | P1 — Safety | Stop all inlet pumps immediately. Notify shift supervisor. |
| V101-TI-01-HH | Blender V101 Temperature High-High | 90°C | P1 — Safety | Shut down agitator & heating. Activate cooling. Evacuate area. |
| P201-ST-01-FAIL | Pump P201 Failure | Run=0 when commanded ON | P2 — Critical | Switch to standby pump P202. Log fault. Call maintenance. |
| T08-LI-01-LL | FG Tank T08 Level Low-Low | 5% fill | P3 — Warning | Alert production scheduler. Reduce fill rate from V102. |
| PLANT-AL-01 | Fire & Gas Detection | Any sensor active | P1 — Safety | Plant emergency shutdown (ESD). Call fire brigade. Evacuate. |

# 4. Data Integration — SCADA to SAP

Batch events from the SCADA historian are passed to SAP PP-PI via a REST API integration layer (Middleware: MuleSoft). Key integration events include: Batch Start, Batch End, Yield Confirmation, QC Hold, and Equipment Downtime.

SAP Endpoint: https://sap-erp.lobp.local/sap/opu/odata/sap/PP_PI_BATCH_SRV/

Authentication: OAuth 2.0 (Client Credentials flow)

Frequency: Event-driven (batch events), 1-minute polling for tank levels

Data Format: OData v4 / JSON