OT CYBERSECURITY RISK ASSESSMENT

Lube Oil Blending Plant — IEC 62443-3-2 Compliance

Document No: CYB-RA-001  |  Rev 1.3  |  Assessment Date: 15-Jul-2024  |  Next Review: Jul-2025

# 1. Assessment Scope & Methodology

This Security Risk Assessment (SRA) covers all Industrial Automation and Control System (IACS) assets at the Lube Oil Blending Plant. It follows the IEC 62443-3-2 methodology: asset identification, threat enumeration, vulnerability assessment, consequence analysis, risk calculation, and countermeasure definition.

Assessor: Cybersecurity team + external consultant (CyberOT Solutions Pvt. Ltd.)

Scope: All OT assets from field PLCs to cloud analytics. Excludes corporate IT (covered by ISO 27001).

Risk Matrix: Likelihood (Low/Medium/High) × Consequence (Medium/High/Critical) = Risk Level

Security Levels: SL 1 (casual) → SL 4 (state-actor). Targets defined per zone below.

# 2. Zone & Conduit Model (IEC 62443-3-2)

| Zone | Assets | Target SL | Key Controls |
| --- | --- | --- | --- |
| Zone 0 — Safety | Safety PLC, ESD system | SL 4 | Air-gapped. No IP connectivity. Physical access control only. |
| Zone 1 — Field | PLCs, sensors, actuators | SL 2 | PROFINET segmented. VLAN isolated. No internet access. |
| Zone 2 — Control | SCADA/HMI, Historian, Engineering WS | SL 2 | Firewall-separated from Zone 3. Jump server required. |
| Zone 3 — Operations | MES, LIMS interface, Batch reporting | SL 2 | DMZ between OT and IT. Bi-directional firewall rules. |
| Zone 4 — Enterprise | SAP ERP, Corporate IT, Email | SL 1 | Standard IT security policies. No direct OT access. |
| Zone 5 — Cloud | Azure IoT Hub, Data Lake, Power BI | SL 2 | TLS 1.3 only. Managed identity. Azure Policy enforced. |

# 3. Risk Register

Risk Level = Likelihood × Consequence.  Critical = immediate action required.  High = action within 30 days.  Medium = action within 90 days.

| ID | Threat Scenario | Threat Source | Likelihood | Consequence | Risk Level | Countermeasures | IEC 62443 Ref | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-001 | Unauthorised remote access to SCADA via VPN | External attacker / disgruntled employee | High | High | Critical | Enforce MFA on VPN. Network segmentation. Jump-server for OT access. | IEC 62443-3-3 SR 1.1 | Open |
| R-002 | Malware introduced via USB on HMI station | Insider / contractor | Medium | High | High | USB port lockdown via endpoint policy. Whitelist only approved devices. | IEC 62443-4-2 CR 7.3 | Closed |
| R-003 | Unpatched PLC firmware — known CVEs | External / supply chain | Medium | High | High | Quarterly patch assessment. Vendor patch advisory subscription. | IEC 62443-2-3 | In Progress |
| R-004 | Replay attack on OPC-UA session | Man-in-middle on OT network | Low | Medium | Medium | OPC-UA Basic256Sha256 + SignAndEncrypt. Session timeout 30 min. | IEC 62443-3-3 SR 4.1 | Closed |
| R-005 | Denial of service on historian causing data loss | External DDoS / misconfiguration | Low | High | High | Edge buffering 72h. Redundant network links. Rate limiting on APIs. | IEC 62443-3-3 SR 7.1 | In Progress |
| R-006 | Tampering with batch formula data in SAP | Insider / privilege escalation | Medium | Critical | Critical | SAP role-based access. Change audit log. Formula approval workflow. | IEC 62443-2-1 | Closed |
| R-007 | Social engineering targeting control room staff | External attacker | Medium | Medium | Medium | Annual cybersecurity awareness training. Phishing simulation exercises. | IEC 62443-2-1 | Open |
| R-008 | Compromise of cloud IoT Hub credentials | Supply chain / credential theft | Low | High | High | Managed identity / no static keys. Azure Defender for IoT enabled. | IEC 62443-3-3 SR 5.2 | Closed |

# 4. Key Findings & Recommendations

## 4.1  Critical Findings

- Unauthorised remote access (R-001): Multi-factor authentication not yet enforced on all VPN accounts. Immediate remediation required.

- Batch formula tampering (R-006): SAP role authorisations reviewed and tightened. Formula change workflow now requires dual approval. CLOSED.

## 4.2  Open Actions

- R-001: MFA rollout scheduled for Q3-2024. Owner: IT Security Manager. Due: 30-Sep-2024.

- R-003: PLC firmware patch assessment in progress. Siemens ProductCERT advisory subscribed. Due: 15-Oct-2024.

- R-007: Cybersecurity awareness training scheduled for all OT staff. E-learning module under development. Due: 30-Nov-2024.